Notes on application MERCURY 1200 (a.k.a. FHR 1200)

Intelligent MODEM

for IMPxx and BYE5xx

Introduction

The MERCURY 1200 is a cheapie HAYES clone which is supplied with

minimal documentation.  The information supplied here is the

results of some experimentation in the application of the MERCURY,

and is intended to help those who are trying to use this modem with

BYE5xx or IMPxx.

Hardware Notes

1)	VERY IMPORTANT!! The nomenclature on the case of some of the

MERCURYs is INCORRECT, and NOT ALL are supplied with an errata

sheet!  From the rear of the device, the connections and switches

ARE (from left to right): POWER, LINE, PHONE, RS-TTL switch,

DCE-DTE switch.  The incorrect case markings transpose the RS-TTL

and DCE-DTE switch!

2)	SW4- Internal DIP switch:

Position 2 is called HCD and is documented as follows:

In the OFF position (default) Carrier Detect operates independent

of the DTR input.  In the ON position Carrier detect is held off

until DTR is presented.

Experimentation has yeilded the following information:

With position 2 in the OFF (default) position.  DTR has NO

effect on MODEM operation.  The modem escape sequence (if

online) followed by the ATH command MUST be used to hang up

the phone!

With position 2 in the ON position. DTR can be used to hang up

the phone and hold off answer.  However, with this setting

the modem will dial, but will not originate a carrier under IMPxx.

The author has made the following mod to his modem:

SW4 position 2 is left open and a separate switch has been run

external to the case and attached in parallel to DIP switch.

I've labelled this ANS/ORIG.  In the OPEN (ORIG) position the

modem will ORIGINATE under IMPxx, but will not use the DTR signal.

In the CLOSED (ANS) position the modem can be used under BYE5xx

using the DTR signal to hang up the phone.

Of course, you can always leave the case off and change the switch

MANUALLY!

REMEMBER to turn modem power OFF when changing any switch setting!!

IMPxx installation

To originate under IMPxxx, the switch discussed above should be

OPEN (in the ORIG mode).  DTR will have no effect on modem operation.

Under IMP244 , use the NODTR feature to inform IMP.  In previous

versions, you'll have to code in your own online ESCAPE and

HANGUP command in the IMP overlay.

BYE5xx installation

Use under BYE is not so straightforward.

USE the ANS mode (SW4 position 2 CLOSED) if possible!!!

USE the following BYE EQUATE settings:

IMODEM	equ	YES	; intellignet modem

B5IM	equ	YES	; MODEM uses protocol

ECHO	equ	NO	; MERCURY does not echo ESC chars,if used

ANCHOR	equ	YES	; MERCURY ACCEPTS the extended commands,

; but IGNORES them.  THE ANCHOR subset is

; sufficient.  ALSO, when answering at

; 300 BAUD it returns the resutl code at

; at 300 (as does the ANCHOR)

HS1200	equ	YES	;

NODTR	equ	NO	; use the DTR line to hang up

NOATA	equ	NO	; MERCURY USES ATA

SHORTB	equ	xx	; doesn't matter, use NO.

NOTE:	If you MUST use NODTR equ YES then you'll have to edit the BYE

source code.  The combination of EQUATES ANCHOR = YES and NODTR = YES

causes BYE to initialize the modem with an escape character of 128d

(80h).  While the MERCURY ACCEPTS the setting, sending three bytes

of 80h (preceeded and followed by a 1 second pause) does not

cause an online escape.  Change the code to eliminate the S2=128

initialization command!  BYE will use the +++ escape sequence.

SET SW4 position 2 to OPEN!

A Special NOTE to Wayne Masters:

The difficulty mentioned in the above paragraph needs your

attention, as the settings ANCHOR=YES and NODTR=YES will currently

cause a combination where the modem (not just the MERCURY) is

programmed as having the escape character as 80h, while BYE issues

an ascii +++.

That's all.

Roger Warren, San Diego, CA.

NOV, '85

#################################################################