;  NBYEUSER.DOC  --  Specifications for the NBYEUSER.INC file for NBYE.MAC

;

;			Gary Case

;			585 Big Sky Court

;			Colorado Springs, CO

;				       80919

;			(303) 599-0744

;

;

;******************************************************************************

;

;

;   NBYE.MAC calls the following 5 routines at the times indicated.  These

;   routines must be located in the NBYEUSER.INC file, and are included to

;   allow special implementation-dependent functions to be performed without

;   cluttering up the NBYE code itself.  This .DOC file is written so it will

;   work as the NBYEUSER.INC file if no special functions are required.

;

;

;******************************************************************************

;

;

;

USRWBOOT:	;This routine is jumped to by NBYE to complete a warm boot.

;When entered, the original BIOS warm boot entry is in HL.

;Normally, this routine consists of a single 'PCHL' to vector

;to the BIOS warm boot point.  The entry is provided for

;special systems which require other processing.

;

;

PCHL			;WARM BOOT ENTIRELY THROUGH THE BIOS

;

;

;

USRINIT:	;This routine is called by NBYE after it has initialized the

;MODEM and is preparing to wait for a call.  If no special

;processing is required, just 'RET'.

;

;	May destroy all registers.

;

;

RET			;NO SPECIAL INITIALIZATION

;

;

;

USRFINI:	;This routine is called when the SYSOP has exited NBYE to CP/M.

;

;	May destroy all registers.

;

;

RET			;NO SPECIAL PROCESSING NEEDED BEFORE EXIT

;

;

;

USRLOGD:	;This routine is called when NBYE has answered the phone,

;received a carrier tone, and successfully logged in a new

;user.

;

;	May destroy all registers.

;

;

RET			;NO SPECIAL PROCESSING NEEDED

;

;

;

USRFUNC:	;This routine is called when the SYSOP has pressed Control-U

;while function keys are enabled.  This allows any special

;functions desired to be implemented without changing NBYE.MAC.

;Note that if additional characters are needed from the local

;console this routine should 'CALL BIOSVECT+9'.  It MUST NOT

;call the 'LCIN' routine, as this would be a recursive call.

;

;	May destroy all registers.

;

;	Must Return Acc=0

;

;

XRA	A		;NO SPECIAL USER FUNCTION IMPLEMENTED

RET			;JUST RETURN WITH ACC=0 PER SPECIFICATION

;

;

;	***  End of Special User Routines  ***

;

;******************************************************************************

##################################################################