ST-Keep System Files Format Reference

Copyright 1991 by Andrew P. Studer

Updated:  07Aug92

* WARNING *  This file contains information critical to the security

of your BBS, do not hand it out freely.

NOTE:  Use this information with extreme caution, I will not be

responsible for loss of data due misuse or inaccuracy of this

document.

****************

*** STK_INFA.SYS - This file contains various defaults for the BBS.

****************

Key:   * = DO NOT MODIFY THIS LINE, SYSTEM CRITICAL INFORMATION

$ = CHANGE ONLY IF YOU MOVE THE CORRESPONDING FILES

Line #  Description

------  -----------

1     Maximum baud rate code:

0 = 19200    1 =  9600    2 =  4800

4 =  2400    7 =  1200    9 =   300

192 = port locked at 19200

2  *  Maximum userlog size

3  *  Maximum number of rooms

4  *  System flags - bit 0 = unused

1 = append new users if userlog full?

2 = allow deletion of msgs in room 4 and

the system activity room?

3 = allow users to create rooms?

4 = install new user accounts?

5  *  System flags - bit 0 = secure mode?

1 = local logon ATS0=0 on / ATH1 off

2 = file transfer verbose?

3 = unused

4 = unused

5 = status line @ logon?

6 = chat mode disabled?

7 = bbs disabled?

6     Number of calls since system initialization

7     Chat exec path/filename

8  *  Maximum number of messages per room

9     System Password

10  $  System files pathname (BBS.PRG & *.SYS files)

11  $  Blurb files pathname (*.BLB)

12  $  Help files pathname (*.HLP)

13  $  Bulletin files (*.BU?)

14  $  Vote files (*.VOT, *.ANS)

15  $  File transfer executable pathname (XYZ.TTP)

16     Pathname for temporary files

17     Post-logon exec door number

18     Unused

19     Door to run if * pressed, null for door menu

20     BBS Name

21     Modem initialization string:

Requires - ATS0=1 to answer on the first ring

ATS2=255 to disable remote control of the modem

DCD should not be set to always on

22     Current message miscellaneous comment field

23     Default time limit, in minutes, for new users, 255 = unlimited

24     Default download ratio for new users, 255 = unlimited

25     SysOp's first name

26     SysOp's last name

27     Logoff Exec door number

28     Unused

29     Room number where ST-Keep will save download messages, 0=room 3

30     Drive/path/filename of STZIP.PRG for verbose listing

31     Drive/path/filename of LHARC.TTP for verbose listing

32     Password to bypass disabled bbs

****************

*** STK_INFB.SYS - Contains message base information

****************

* WARNING *  Never edit this file or you will destroy the integrity

of your message base.

Line #  Description

------  -----------

1     Total number of bytes written to the message file

2     Oldest message number in the message file

3     Number of the next message to be written to the message file

4     File offset location to write the next message

****************

*** STK_INFC.SYS - Contains ST-Keep color palette data

****************

32 bytes of information, 2 bytes for each color register (0-15) used

by the ST's setpalette() routine (XBIOS #6).

****************

*** STK_HDRS.SYS - Room header information

****************

Each line in this file contains the room header for the relative room

number.  Line 1 contains the header for the Lobby, line 2 for Mail,

etc.  [R][I] shows the room number you are in.  If used, this file

must contain a number of lines equal to the maximum number of rooms

in your system with blank lines for rooms without headers.

***************

*** STK_ARC.SYS - Room archive filenames

***************

Each line in this file contains the pathname/filename of each room

that is being archived.  If used, this file must contain a number of

lines equal to the maximum number of rooms in your system with blank

lines for rooms that aren't being archived.

***************

*** STK_GRP.SYS - Group names

***************

15 lines, each containing the name of the relative group.  If the

group is unused the line must be blank.

****************

*** STK_ROOM.SYS - Room data

****************

This block of data is repeated a number of times equal to the maximum

number of rooms.

Offset    Description

--------  -----------

1 - 30   Name, space padded to the right

31 - 60   Directory pathname, space padded to the right

61        Generation number, 0=inactive, 1-255 gen

62        First flag

63        Second flag

64        Group number, if non-zero this room is only available to

users of this group

65        Floor number that it exists on

The following two fields are repeated for each message in

the room.  If your system has 50 messages per room, this

group of bytes will be repeated 50 times before the next

room information structure begins.

66 - 69   Message number

70 - 73   Message location

****************

*** STK_USER.SYS - User account information

****************

This block of data is repeated a number of times equal to the maximum

number of user accounts.

Offset    Description

--------  -----------

1 - 20   Password

21 - 50   Username

51 - 70   Personal Comment field

71 - 83   Phone Number

84        Access Level

85 - 86   Number of calls (hi/low)

87 - 88   Last call date (hi/low MS/DOS format)

89 - 90   Last call time (hi/low MS/DOS format)

91 - 100  Users birth date (mm/dd/yyyy format)

101       Unused

102       Minutes per day

103       Minutes left today

104       Screen width

105       Userflag A

106       Userflag B

107 - 117 Unused

118 - 199 Download count (hi/low)

120 - 121 Upload count (hi/low)

122       Upload protocol

123 - 124 Number of messages written (hi/low)

125       Download ratio

126 - 127 Unused

128       Graphic emulation (0=ascii, 1=vt52, 2=ansi)

129 - 130 Group bits (0-15)

131       Tab spaces (0+)

132 - 134 Unused

135 - 164 Real Name

165 - 184 City, State

The following two fields are repeated a number of times

equal to the maximum number of messages allowed in each

room.  These are user mail pointers.

185 - 188 Message number

189 - 192 Message location

The following two fields are repeated once for every room

on the BBS.  This includes unused room slots.

193       Generation number for this room

194 - 197 Last message read in this room

"     "  Next user structure