[H[2J  Frequently Asked Questions about X2 Technology (x2.usr.com/faq.html)

Q: What is x2 Technology?

A: x2 is the new method developed by

U.S. Robotics for 56K downloads from the Internet

and online services over regular phone lines.

*x2 is capable of downloading at 56 Kbps. However,

current FCC regulations limit download speed to 53

Kbps.

Q: What about uploads?

A: x2 downloads (receives data) at up to 56Kbps. It

uploads (sends data) at up to 33.6Kbps

Q: Why doesn't x2 go 56K in both directions?

A: x2 Technology is asymmetric. It sends and

receives at different speeds to take advantage of the

digital connections most ISPs (Internet service

providers) have to the public phone network. Note

that V.34 is also asymmetric; it can increase speed in

one or the other direction, as line conditions allow.

Q: Don't phone lines max out at about 35K? (Isn't the theoretical

limit for data transmission over phone lines somewhere around

35K?) (Shannon's Law)

A: Yes. But if one assumes analog connections to the

phone network on both ends. The actual bandwidth

of a pure digital channel like ISDN is 64K. By taking

advantage of the digital connections (for example,

T1) that most ISPs have to the phone network, x2

provides download connect rates of up to 56K over

regular phone lines.

Q: How does it do that?

A: x2 Technology takes advantage of a special

network configuration found when an analog client

connects to a digitally connected server. By

eliminating the analog-to-digital conversion in the

downstream path, x2 can use the practical maximum

amount of the available 64 Kbps bandwidth. This

technique makes modems with x2 fundamentally

different from other high-speed modems. In fact,

downloads could more accurately be described as

encoded, rather than modulated.

Q: How does x2 compare to ISDN and other faster technologies?

A: x2 provides high speed performance nearly as

fast as ISDN connections, without the added

expense of complex installation and high monthly

fees associated with ISDN. Other technologies, such

as ADSL or cable modems, are in the very early

development stages and are not widely available. x2

will be available in early 1997, and will use regular

phone lines for high-speed connections.

Q: Does x2 Technology work over regular phone lines?

A: Yes. x2 can provide downstream connect rates of

up to 56K over regular phone lines. However,

current FCC regulations limit download speed to 53

Kbps.

Q: What if I don't get 28.8 connections on my current phone line?

Will x2 still work?

A: Yes. We expect x2 will work on the vast majority

of North American phone lines at download speeds

from 32K up to 53Kbps even if your usual V.34

connect speed is below 28.8K.

Q: Does x2 work through PBXes?

A: Probably not. x2 requires a direct digital path

from the ISP to the desktop modem. Most PBXes

introduce additional digital-to-analog conversions,

which reduce x2's performance.

Q: Is x2 a compression method?

A: No. x2 provides actual connect rates of up to

56K*. Data compression (V.42 bis) can increase

throughput to up to 115.2 Kbps.

Q: Can two Sportster modems connect to each other at 56K?

A: No. x2 requires an x2-capable server digitally

connected to the phone network, for example, a

Total Control Enterprise Network Hub linked by

T1 or PRI. Only desktop modems such as the

Courier I-modem will be able to act as digital

servers for 56K Technology. Desktop modems

connected to regular phone lines will not be able to

act as x2 servers.

Q: Can I upgrade my modem to support x2 Technology?

A: Yes. All Courier and Sportsterr 33.6 desktop

modems on retail shelves today are upgradable to

x2. Any Winmodem is upgradable and any other

desktop Sportster purchased after August 15, 1996

is also upgradable.

Q: Are there any distance limitations regarding x2?

A: While a person's distance from the telephone

company's central office is one of the variables which

could affect the connect speed, other variables play

a role as well. In addition, there is virtually no way

for the subscriber to obtain loop length or other data

regarding their POTS line. While more severe loop

impediments would result in a connection below 56

Kbps, the subscriber would still typically connect at

a high rate (faster than V.34 speeds).

Q: Since maximum speeds are seldom attained, what realistic

speed improvements should end users expect to see?

A: While the answer to this question is dependent

upon a case-by-case analysis, subscribers will

typically see speed improvements using x2

Technology in comparison to traditional analog

technologies. In fact, trials involving thousands of

calls in a multitude of regions have shown that a

preponderance of those calls were able to achieve

the faster x2 speeds.

x2 is a different protocol than today's analog

modulation schemes. These latter schemes assume

an analog connection end-to-end. In contrast, x2

Technology assumes a primarily digital connection,

with an analog leg needed only to connect the

subscriber modem. For that reason, customers who

today cannot even achieve V.34 speeds will more

than likely be able to connect via the higher x2

speeds. In addition, there are regions where the

serving telephone company has declared that their

subscribers will not even be able to reach V.34

speeds. Nevertheless, subscribers in these regions

will typically achieve the higher x2 speeds because

the telephone company has assumed in their

statement an analog end-to-end connection

employing the older analog modem standards.

Q: What are the benefits of x2 as compared to ISDN?

A: While there will be applications for which the

customer requires ISDN services rather than x2

functionality, x2 offers the average consumer the

ability to take advantage of near-ISDN speeds

quickly and cost-effectively.

Because x2 uses the standard analog telephone

service consumers already have today, subscribers

will find x2 easy to implement. ISDN also requires

the use of special equipment called Terminal

Adapters, whereas x2 requires analog modems,

which are quite a bit cheaper. Furthermore, analog

service is cheaper than ISDN. Finally, analog service

is readily available everywhere, while ISDN may not

be easily available in all areas.

Q: What's the current status of x2 within the standards bodies?

A: U.S. Robotics submitted the x2 proposal to

ITU-T for consideration on September 27, 1996. x2

was the first proposal for the standardization of

56-Kbps analog modem technology.

Q: Is there an efficient way to determine if the telco, ISP or

geographic region has the correct network to support x2?

A: One requirement for x2 Technology is a digital

connection at the server end. Because of the digital

nature of today's public telephone network, these

interfaces are readily available throughout the United

States. To provide a greater level of detail, U.S.

Robotics has data compiled by Bellcore which

assesses the availability of the digital interfaces

offered in the different regions across the United

States.

Q: What is involved with upgrading equipment to support x2

functionality?

A: x2 will be offered in all Network Systems

products (Total Control, MP, NETServer and

Courier) as a software download. This means the

customer merely has to upgrade their current

equipment's software, rather than having to actually

replace hardware. The process will be simple and

inexpensive. The Sportster Winmodem products are

software upgradable. Sportster 33.6 desktop

products purchased after August 15, 1996 can be

upgraded to x2 by swapping the product's ROM

integrated circuit. Details on upgrades for the specific

U.S. Robotics products will be available prior to the

availability of x2 Technology.

Q: Once it's upgraded, will my modem still talk with all of

the other modems out there?

A: Yes. x2-capable modems will be backward

compatible with all the other existing modem

standards in V.Everythingr. However, two

client-side x2 modems cannot achieve near 56-Kbps

speeds! For that, an x2 analog modem must talk to

server-side equipment that connects to the public

network via a digital interface.

Q: Are all Courier modems upgradable?

A: All desktop Courier V.34 or V.Everything

modems are upgradable to x2. This also includes the

Dual Standard HST products that were marketed by

U.S. Robotics, as long as they are first upgraded to

V.Everything via the available daughter card upgrade

(U.S. Robotics SKU number 001225-0).

Q: Will the Courier-I-modem use x2 at both the client and server

ends?

A: Yes. A Courier modem or Courier I-modem

operating in client mode can connect to a Courier

I-modem operating in server mode.

Q: Will the MP/8 I-modem and NETServer/8 I-modem products

support 8 simultaneous x2 calls or just 4?

A: x2 calls are analog calls and thereby occupy one

channel. Therefore, the MP/8-I Modem and

NetServer/8-I modem products will each support

eight x2 calls.

Q: Will T3s work as the digital trunk?

A: Yes, but you will need some form of a T3

multiplexer on the front end of the U.S. Robotics

POP equipment to demultiplex the DS3 into the 28

DS1's riding within that DS3. The DS3 multiplexing

process will not impact the x2 functionality.

Q: What effect will T1 over HDSL have on x2 performance?

A: HDSL is a technique for transmitting T1/E1 digital

services. Since it maintains the bit patterns of the

original signal, it will not have an effect on x2

performance.

Q: Can Fractional T1 support x2?

A: Yes. x2 channels are still transported digitally

within the fractional T1.

Q: What effect will a DS1 over a microwave hop have on x2

performance?

A: None. Given that a DS1 transmitted over a

microwave link will need to adhere to the same

specifications such as bit error rate as over other

media types, this choice of DS1 digital transmission

technology will not have an effect on the

performance of x2 Technology.